Intel CPU Sockets and Compatible Processors

Intel uses Land Grid Array (LGA) sockets for most modern desktop and server processors, with pin counts varying by generation. Below is a list of major sockets, focusing on desktop and some server types, along with compatible processor families. Compatibility often requires matching chipsets and may need BIOS updates.[1][2][3][4][5][6]

| Socket | Launch Year | Compatible Processors |
| --- | --- | --- |
| LGA 775 | 2004 | Core 2 Duo, Core 2 Quad, Pentium Dual-Core, Celeron (45nm and 65nm)[1][3] |
| LGA 1156 | 2009 | 1st Gen Core i3/i5/i7 (Nehalem, Westmere)[1][3] |
| LGA 1155 | 2011 | 2nd/3rd Gen Core i3/i5/i7 (Sandy Bridge, Ivy Bridge)[1][3] |
| LGA 1150 | 2013 | 4th/5th Gen Core i3/i5/i7 (Haswell, Broadwell)[1][3] |
| LGA 1151 | 2015 | 6th/7th Gen Core i3/i5/i7 (Skylake, Kaby Lake); 8th/9th Gen (Coffee Lake) with 300-series chipsets[1][2][4] |
| LGA 1200 | 2020 | 10th/11th Gen Core i3/i5/i7/i9 (Comet Lake, Rocket Lake)[1][3][6] |
| LGA 1700 | 2021 | 12th/13th/14th Gen Core i3/i5/i7/i9 (Alder Lake, Raptor Lake)[3][6] |
| LGA 1851 | 2024 | Core Ultra 200 series (Arrow Lake)[7] |
| LGA 2066 | 2017 | Core i7/i9 X-series (Skylake-X, Cascade Lake-X)[3] |
| LGA 2011 | 2011 | Core i7 Extreme (Sandy Bridge-E to Broadwell-E); Xeon E5/E7 families[3][5] |
| LGA 3647 | 2017 | Xeon Scalable (1st/2nd Gen)[5] |
| LGA 4189 | 2020 | 3rd Gen Xeon Scalable[5] |
| LGA 4677 | 2022 | 4th/5th Gen Xeon Scalable[7][5] |
| LGA 4710 | 2023 | Xeon 6 (Granite Rapids)[7][5] |

AMD CPU Sockets and Compatible Processors

AMD sockets include AM, FM, and TR series for desktop, with variations for servers and mobile. Compatibility depends on chipset and BIOS; AM4/AM5 offer broad support across Ryzen generations.[8][9][10][11]

| Socket | Launch Year | Compatible Processors |
| --- | --- | --- |
| Socket A (462) | 2000 | Athlon XP, Sempron (early models)[8][9] |
| Socket AM2 | 2006 | Athlon 64, Phenom (DDR2 support)[8][9] |
| Socket AM2+ | 2007 | Phenom II, Athlon II (backward compatible with AM2)[8][9] |
| Socket AM3 | 2009 | Phenom II, Athlon II, Sempron (DDR3 support; some AM3+ compatibility with BIOS)[8][9] |
| Socket AM3+ | 2011 | FX series (Bulldozer, Vishera), backward compatible with AM3[8][9] |
| Socket AM4 | 2017 | Ryzen 1000-5000 series (Zen to Zen 3), Athlon (with APU variants); supports up to Ryzen 9000 with BIOS[8][9][11] |
| Socket AM5 | 2022 | Ryzen 7000-9000 series (Zen 4 and later; DDR5 only)[7][9] |
| Socket FM1 | 2011 | A-series APUs (Llano)[8][9] |
| Socket FM2 | 2012 | A-series APUs (Trinity, Richland)[8][9] |
| Socket FM2+ | 2014 | A-series APUs (Kaveri, Godavari)[8][9] |
| Socket TR4 (sTRX4) | 2017 | Ryzen Threadripper 1000-3000 series[9] |
| Socket sTR5 | 2023 | Ryzen Threadripper 7000 series[7][9] |
| Socket SP3 | 2017 | EPYC 7000-8000 series (server)[9] |
| Socket SP5 | 2022 | EPYC Genoa (9000 series)[7][9] |
| Socket SP6 | 2023 | EPYC Siena[7][9] |

<div style="text-align: center">⁂</div>

- https://www.buildcomputers.net/intel-cpu-socket.html

- https://en.wikipedia.org/wiki/Category:Intel_CPU_sockets

- https://www.intel.com/content/www/us/en/support/articles/000005670/processors.html

- https://www.intel.com/content/www/us/en/support/articles/000027816/processors/intel-core-processors.html

- https://www.intel.com/content/www/us/en/support/articles/000055173/processors/intel-xeon-processors.html

- https://www.intel.com/content/www/us/en/support/articles/000092149/processors.html

- https://en.wikipedia.org/wiki/CPU_socket

- https://www.buildcomputers.net/amd-cpu-socket.html

- https://en.wikichip.org/wiki/amd/List_of_AMD_CPU_sockets

- https://en.wikipedia.org/wiki/Category:AMD_sockets

- https://www.amd.com/en/products/processors/chipsets/am4.html

- https://www.gigabyte.com/Support/CPU-Support

- https://vstl.info/amd-cpu-compatibility-guide/

- https://www.amd.com/en/products/processors/desktops/ryzen.html

- https://www.intel.com/content/www/us/en/products/details/processors/core.html

- https://www.reddit.com/r/buildapc/comments/10q43oj/dumb_question_intel_cpu_generations_compatibility/

- https://www.cclonline.com/pc-components/cpu-processors/amd-socket-am4-processors/

- https://learn.microsoft.com/en-us/windows-hardware/design/minimum/supported/windows-11-supported-intel-processors

- https://www.amd.com/en/products/specifications/processors.html

- https://en.wikipedia.org/wiki/Template:AMD_CPU_sockets